# Article 5	Commercial and MIXED-USE Zoning Districts

## 5.01	RHB - Rural Hub Zoning District

- Purpose: The rural hub zoning district allows for limited retail and service uses and activities that offer convenience to and support nearby rural residential development. These areas are typically located at or near intersections of arterial and/or collector streets in order to maximize accessibility from surrounding areas. Site design and performance standards help maximize compatibility between this district and surrounding rural areas and reinforce vehicular and pedestrian accessibility within and between these zones.

- Typical Development Pattern.

- Uses and Activities Allowed.

      - Uses and activities allowed in the RHB zoning district are established in Article 9 of this ordinance as permitted or conditionally permitted. Reference the consolidated table of uses for additional information.

For ease in review of these articles, a temporary list of permitted and conditional uses allowed within the RHB zoning district has been provided in Table 5.01.01 below.

      - Multiple principal and accessory uses and structures may be permitted on a lot within this zoning district. Multiple uses may be allowed within a single structure.

- Table 5.01.02 Lot Dimensions. (Figure 5.01.01 Lot Dimensions)

|  | Min. Lot Area (ac) | Max. Gross Floor Area (sq. ft.) | Min. Lot Width (ft) | Width to Depth Ratio (%) | Max. Impervious Surface (%) |
| --- | --- | --- | --- | --- | --- |
| All types | 0.5 | 20,000 | 100 | n/a | 45 |
| Key | C | D | A | A/B | - |

- Table 5.01.03 Lot Setbacks. (Figure 5.01.02 Setbacks and Build-To Zones)

|  | Min. Front (ft) | Min. Rear (ft) | Min. Side, Interior (ft) | Min. Side, Street (ft) | Interior (ft. btwn buildings if multiple) |
| --- | --- | --- | --- | --- | --- |
| Principal Structure(s) | 20* | 10^ | 10^ | 20* | No minimum |
| Accessory Structure(s) | 20* | 10^ | 10^ | 20* | No minimum |
| Key | A | B | C | D | E |

^Setback increases to 35 feet if abutting a residential zoning district or lot with an established residential use.

*Street frontage type may dictate a reduced setback; refer to Article 18 for additional detail.

- Table 5.01.04 Build-to Zone. (Figure 5.01.02 Setbacks and Build-To Zones)

There is no build-to zone established or regulated in the RHB district.

|  | Front (%) | Key | Side Street (%) | Key |
| --- | --- | --- | --- | --- |
| Principal Structure(s) | n/a | F | n/a | G |
| Accessory Structure(s) | n/a | F | n/a | G |

- Table 5.01.05 Building Height. (Figure 5.01.03 Building Height)

|  | Max Height (ft) | Key | Max Height, First Story (ft) | Key |
| --- | --- | --- | --- | --- |
| Principal Structure(s) | 35 | A | n/a | B |
| Accessory Structure(s) | 35 | C | n/a | D |

- Table 5.01.06 Access.

Access standards are still under development in coordination with updates to the Knox County Access Policy. This section will be updated following completion of draft Articles 13 and 18 and further guidance provided by policy adjustment.

- Table 5.01.07 Parking. (Figure 5.01.04 Parking)

| Surface Parking | Surface Parking | Surface Parking |
| --- | --- | --- |
| Location | Permitted (y/n) | Key |
| Front yard | Y | A |
| Side yard (interior) | Y | B |
| Side yard (street) | Y | C |
| Rear yard | Y | D |
| Minimum Parking Setback from Property Boundary (ft) | Minimum Parking Setback from Property Boundary (ft) | Minimum Parking Setback from Property Boundary (ft) |
| Front^ | 10 | A |
| Side (interior) | 5* | B |
| Side (street) | 10 | C |
| Rear^ | 5* | D |

^Does not apply to driveway surface parking.

*Parking setback increases to 15 feet when adjacent to a residential district or established residential use.

- Infrastructure.

      - Maximum impervious surface coverage allowed may be increased through the use of pervious pavement. Reference Article 13 for requirements applicable to this district.

- Landscaping.

No district-specific infrastructure requirements have currently been identified. Moving forward, this section will be used to identify district-specific requirements if they are needed.

- Open Space.

      - The required open space set-aside in an RHB zoning district can be found in Article 16 of this ordinance.

- Additional Considerations.

No additional considerations have currently been identified. Moving forward, this section will be used to point to specific requirements if they are needed.

## 5.02	NMU - Neighborhood Mixed-Use Zoning District

- Purpose: The neighborhood mixed-use zoning district allows for office, retail, and service uses and activities that are compatible with and meet the needs of established neighborhoods. Limited residential uses are compatible. Areas zoned neighborhood mixed-use are typically located at or near intersections with a collector or arterial street, at the edge of or entrance to an established neighborhood. Site design and performance standards help maximize compatibility between this district and the adjacent neighborhoods and reinforce vehicular and pedestrian accessibility within and between neighborhoods.

- Typical Development Pattern.

- Uses and Activities Allowed.

      - Uses and activities allowed in the NMU zoning district are established in Article 9 of this ordinance as permitted or conditionally permitted. Reference the consolidated table of uses for additional information.

For ease in review of these articles, a temporary list of permitted and conditional uses allowed within the NMU zoning district has been provided in Table 5.02.01 below.

      - Multiple principal and accessory uses and structures may be permitted on a lot within this zoning district. Multiple uses may be allowed within a single structure.

- Table 5.05.02 Lot Dimensions. (Figure 5.02.01 Lot Dimensions)

| Building Type | Min. Lot Area (ac) | Max. Gross Floor Area (sq. ft.) | Min. Lot Width (ft) | Min. Lot Width (ft) w/Alley | Width to Depth Ratio (%) | Max. Impervious Surface (%) |
| --- | --- | --- | --- | --- | --- | --- |
| Mixed-use horizontal | No minimum | -- | 50 | 40 | 1:2 | 80 |
| Key | C | D | A | A | A/B | - |

- Table 5.02.03 Lot Setbacks. (Figure 5.02.02 Lot Setbacks and Build-To Zone)

|  | Min. Front (ft) | Min. Rear (ft) | Min. Side, Interior (ft) | Min. Side, Street (ft) | Interior (ft. btwn buildings if multiple) |
| --- | --- | --- | --- | --- | --- |
| Principal Structure(s) | 5* | 10^ | No minimum* | 5* | No minimum |
| Accessory Structure(s) | 10* | 10^ | 5* | 10* | No minimum |
| Key | A | B | C | D | E |

^Setback increases to 35 feet if abutting a residential zoning district or lot with an established residential use.

*Street frontage type may dictate a reduced setback; refer to Article 18 for additional detail.

- Table 5.02.04 Build-to Zone. (Figure 5.02.02 Lot Setbacks and Build-To Zone)

|  | BTZ - Front (ft) | Front (%) | Key | BTZ – Side Street (ft) | Side Street (%) | Key |
| --- | --- | --- | --- | --- | --- | --- |
| Principal Structure(s) | 10 | 50^ | F | 10 | 40^ | G |
| Accessory Structure(s)* | None established | n/a | F | 20 | 40 | G |

^When multiple principal structures are present on a lot or development site, the % façade requirement within the established build-to zone reflects the combined total amount of façade required in the build-to zone.

*Must be located behind the front façade line of a principal structure.

- Table 5.02.04 Building Height. (Figure 5.02.03 Building Height)

|  | Max Height (ft) | Key | Max Height, First Story (ft) | Key |
| --- | --- | --- | --- | --- |
| Principal Structure(s) | 35 | A | 12 | B |
| Accessory Structure(s) | 35 | C | n/a | D |

- Table 5.02.06 Access.

Access standards are still under development in coordination with updates to the Knox County Access Policy. This section will be updated following completion of draft Articles 13 and 18 and further guidance provided by policy adjustment.

- Table 5.02.07 Parking. (Figure 5.02.04 Parking)

| Surface Parking | Surface Parking | Surface Parking |
| --- | --- | --- |
| Location | Permitted (y/n) | Key |
| Front yard | N | A |
| Side yard (interior) | Y | B |
| Side yard (street) | N | C |
| Rear yard | Y | D |
| Minimum Parking Setback from Property Boundary (ft) | Minimum Parking Setback from Property Boundary (ft) | Minimum Parking Setback from Property Boundary (ft) |
| Front^ | n/a | A |
| Side (interior) | 5* | B |
| Side (street) | n/a | C |
| Rear^ | 5* | D |

^Does not apply to driveway surface parking.

*Parking setback increases to 15 feet when adjacent to a residential district or established residential use.

- Infrastructure.

No district-specific landscaping requirements have currently been identified. Moving forward, this section will be used to identify district-specific requirements if they are needed.

- Landscaping.

No district-specific landscaping requirements have currently been identified. Moving forward, this section will be used to identify district-specific requirements if they are needed.

- Open Space.

      - The required open space set-aside in an NMU zoning district can be found in Article 16 of this ordinance.

- Additional Considerations.

  - Development within the NMU district is required to incorporate a blend of building types and uses.

    - A lot or development site two acres or greater requires a minimum of 20% of the developed gross floor area to be non-residential

    - Development shall be configured to locate non-residential building types and uses at designated intersections within the development site.

    - Ground floor residential is not permitted in the shopfront or mixed-use building types.

## 5.03	MUCC - Mixed-Use Community Corridor Zoning District

- Purpose: The mixed-use community corridor zoning district is established to create and enhance vibrant, pedestrian-oriented mixed-use corridors along major thoroughfares. This district accommodates a mix of residential, retail, and service uses and activities that serve the needs of adjacent neighborhoods as well as the broader community. The district supports the development and redevelopment of underutilized commercial land and is characterized by a variety of building and development types that may include vertical mixed-use within a building or a horizontal mix of uses in one or multiple buildings development-wide. Site design standards ensure that development is supported by transportation infrastructure that safely connects cars and pedestrians to and through the district and ensures compatibility between mixed-use development and adjacent residential areas.

- Typical Development Pattern.

- Uses and Activities Allowed.

      - Uses and activities allowed in the MUCC zoning district are established in Article 9 of this ordinance as permitted or conditionally permitted. Reference the consolidated table of uses for additional information.

For ease in review of these articles, a temporary list of permitted and conditional uses allowed within the MUCC zoning district has been provided in Table 5.03.01 below.

      - Multiple principal and accessory uses and structures may be permitted on a lot within this zoning district. Multiple uses may be allowed within a single structure.

- Table 5.03.02 Lot Dimensions. (Figure 5.03.01 Lot Dimensions)

|  | Min. Lot Area (ac) | Max. Gross Floor Area (sq. ft.) | Min. Lot Width (ft) | Width to Depth Ratio (%) | Max. Impervious Surface (%) |
| --- | --- | --- | --- | --- | --- |
| All types | No minimum | -- | 50 | n/a | 80 |
| Key | C | D | A | A/B | - |

- Table 5.03.03 Lot Setbacks. (Figure 5.03.02 Setbacks and Build-To Zones)

| Building Type | Min. Front (ft)* | Min. Front (ft) w/Alley | Min. Rear (ft)^ | Min. Rear (ft.) w/Alley^ | Min. Side, Interior (ft)** | Min. Side, Street (ft)* | Interior (ft. btwn buildings if multiple) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Multifamily (9+ units) | 10 | No minimum | 5 | 5 | 5 | No minimum | 10 |

^Setback increases to 20 feet for the portion of structures above 35 feet if abutting residential zoning district or an established residential use.

* Street frontage type may dictate a reduced setback; refer to Article 18 for additional detail.

**Shared walls between units do not have a side setback.

- Table 5.03.04 Build-to Zone. (Figure 5.03.02 Setbacks and Build-To Zones)

|  | BTZ - Front (ft) | Front (%) | Key | BTZ – Side Street (ft) | Side Street (%) | Key |
| --- | --- | --- | --- | --- | --- | --- |
| Principal Structure(s) | 15 | 60^ | F | 15 | 50^ | G |
| Accessory Structure(s)* | None established | n/a | F | 15 | 40 | G |

^When multiple principal structures are present on a lot or development site, the % façade requirement within the established build-to zone reflects the combined total amount of façade required in the build-to zone.

*Must be located behind the front façade line of a principal structure.

- Table 5.03.05 Building Height. (Figure 5.03.03 Building Height)

|  | Max Height (ft) | Key | Max Height, First Story (ft) | Key |
| --- | --- | --- | --- | --- |
| Principal Structure(s) | 60 | A | 14 | B |
| Accessory Structure(s) | 25 | C | n/a | D |

- Table 5.03.06 Access.

Access standards are still under development in coordination with updates to the Knox County Access Policy. This section will be updated following completion of draft Articles 13 and 18 and further guidance provided by policy adjustment.

- Table 5.03.07 Parking. (Figure 5.03.04 Parking)

| Surface Parking | Surface Parking | Surface Parking |
| --- | --- | --- |
| Location | Permitted (y/n) | Key |
| Front yard | N | A |
| Side yard (interior) | Y | B |
| Side yard (street) | N | C |
| Rear yard | Y | D |
| Minimum Parking Setback from Property Boundary (ft) | Minimum Parking Setback from Property Boundary (ft) | Minimum Parking Setback from Property Boundary (ft) |
| Front^ | n/a | A |
| Side (interior) | 5* | B |
| Side (street) | n/a | C |
| Rear^ | 5* | D |

^Does not apply to driveway surface parking.

*Parking setback increases to 15 feet when adjacent to a residential district or established residential use.

- Infrastructure.

No additional infrastructure requirements have currently been identified. Moving forward, this section will be used to point to specific requirements if they are needed.

- Landscaping.

No district-specific infrastructure requirements have currently been identified. Moving forward, this section will be used to identify district-specific requirements if they are needed.

- Open Space.

- The required open space set-aside in an MUCC zoning district can be found in Article 16 of this ordinance.

- Additional Considerations.

No additional considerations have currently been identified. Moving forward, this section will be used to point to specific requirements if they are needed.

## 5.04	HC - Highway Commercial Zoning District

- Purpose: The highway commercial zoning district provides commercial activities that cater to the needs of the highway user and the general public. This district is designed to accommodate a range of uses that require a high degree of visibility and accessibility from major arterial streets and state and local highways.

- Typical Development Pattern.

- Uses and Activities Allowed.

      - Uses and activities allowed in the HC zoning district are established in Article 9 of this ordinance as permitted or conditionally permitted. Reference the consolidated table of uses for additional information.

For ease in review of these articles, a temporary list of permitted and conditional uses allowed within the HC zoning district has been provided in Table 5.04.01 below.

      - Only one principal use or structure is permitted on a lot within this zoning district. Multiple uses may be allowed within a single structure. More than one use or structure considered accessory to the principal use or structure may be permitted.

- Table 4.04.2 Lot Dimensions. (Figure 4.04.01 Lot Dimensions)

|  | Min. Lot Area (sf) | Min. Lot Width (ft) | Width to Depth Ratio (%) | Max. Impervious Surface (%) |
| --- | --- | --- | --- | --- |
| All types | No minimum | 100 | 1:2 | 75 |
| Key | C | A | A/B | - |

- Table 4.04.03 Lot Setbacks. (Figure 4.04.02 Lot Setbacks and Build-To Zone)

|  | Min. Front (ft) | Min. Rear (ft) | Min. Side, Interior (ft) | Min. Side, Street (ft) | Interior (ft. btwn buildings if multiple) |
| --- | --- | --- | --- | --- | --- |
| Principal Structure | 20* | 5^ | 5 | 10* | 24 |
| Accessory Structure(s) | 20* | 5^ | 5 | 10* | 24 |
| Key | A | B | C | D | E |

^Setback increases to 20 feet if abutting a residential zoning district or lot with an established residential use.

*Street frontage type may dictate a reduced setback; refer to Article 18 for additional detail.

- Table 5.04.04 Build-to Zone. (Figure 5.04.02 Setbacks and Build-To Zones)

There is no build-to zone established or regulated in the CH district.

|  | Front (%) | Key | Side Street (%) | Key |
| --- | --- | --- | --- | --- |
| Principal Structure | n/a | F | n/a | G |
| Accessory Structure(s) | n/a | F | n/a | G |

- Table 5.04.05 Building Height. (Figure 5.04.03 Building Height)

|  | Max Height (ft) | Key | Max. Height, First Story (ft) | Key |
| --- | --- | --- | --- | --- |
| Principal Structure | 60 | A | n/a | B |
| Accessory Structure(s) | 25 | C | n/a | D |

- Table 5.04.06 Access.

Access standards are still under development in coordination with updates to the Knox County Access Policy. This section will be updated following completion of draft Articles 13 and 18 and further guidance provided by policy adjustment.

- Table 5.04.07 Parking. (Figure 5.04.04 Parking)

| Surface Parking | Surface Parking | Surface Parking |
| --- | --- | --- |
| Location | Permitted (y/n) | Key |
| Front yard | Y | A |
| Side yard (interior) | Y | B |
| Side yard (street) | Y | C |
| Rear yard | Y | D |
| Minimum Parking Setback from Property Boundary (ft) | Minimum Parking Setback from Property Boundary (ft) | Minimum Parking Setback from Property Boundary (ft) |
| Front^ | 5 | A |
| Side (interior) | 5* | B |
| Side (street) | 0 | C |
| Rear^ | 5* | D |

^Does not apply to driveway surface parking.

*Parking setback increases to 15 feet when adjacent to a residential district or established residential use.

- Infrastructure.

No additional infrastructure requirements have currently been identified. Moving forward, this section will be used to point to specific requirements if they are needed.

- Landscaping.

No district-specific infrastructure requirements have currently been identified. Moving forward, this section will be used to identify district-specific requirements if they are needed.

- Open Space.

- The required open space set-aside in an HC zoning district can be found in Article 16 of this ordinance.

- Additional Considerations.

No additional considerations have currently been identified. Moving forward, this section will be used to point to specific requirements if they are needed.

## 5.05	BC - Business and Commercial Zoning District

- Purpose: The business and commercial zoning district provides for professional office space, medical offices and clinics, retail, and services at a smaller scale than the employment campus environment. Such uses and activities that fall under this zone are typically located on their own lot or share a common building.

- Typical Development Pattern.

- Uses and Activities Allowed.

      - Uses and activities allowed in the BC zoning district are established in Article 9 of this ordinance as permitted or conditionally permitted. Reference the consolidated table of uses for additional information.

For ease in review of these articles, a temporary list of permitted and conditional uses allowed within the BC zoning district has been provided in Table 5.04.01 below.

      - Only one principal use or structure is permitted on a lot within this zoning district. Multiple uses may be allowed within a single structure. More than one use or structure considered accessory to the principal use or structure may be permitted.

- Table 5.05.02 Lot Dimensions. (Figure 5.05.01 Lot Dimensions)

|  | Min. Lot Area (ac) | Gross Floor Area (sq. ft.) | Min. Lot Width (ft) | Width to Depth Ratio (%) | Max. Impervious Surface (%) |
| --- | --- | --- | --- | --- | --- |
| All types | No minimum | - | 80 | n/a | 65 |
| Key | C | D | A | A/B | - |

- Table 5.05.03 Lot Setbacks. (Figure 5.05.02 Setbacks and Build-To Zone)

|  | Min. Front (ft) | Min. Rear (ft) | Min. Side, Interior (ft) | Min. Side, Street (ft) | Interior (ft. btwn buildings if multiple) |
| --- | --- | --- | --- | --- | --- |
| Principal Structure | 25* | 10^ | 10^ | 25* | 24 |
| Accessory Structure(s) | 25* | 5^ | 5^ | 25* | 24 |
| Key | A | B | C | D | E |

^Setback increases to 20 feet if abutting a residential zoning district or lot with an established residential use.

*Street frontage type may dictate a reduced setback; refer to Article 18 for additional detail.

- Table 5.05.04 Build-to Zone. (Figure 5.05.02 Setbacks and Build-To Zone)

| Building Types | BTZ - Front (ft) | Front (%) | Key | BTZ – Side Street (ft) | Side Street (%) | Key |
| --- | --- | --- | --- | --- | --- | --- |
| Principal Structure | 35 | 15 | F | 35 | 10 | G |
| Accessory Structure(s) | None established | n/a | F | None established | n/a | G |

- Table 5.05.05 Building Height. (Figure 5.05.03 Building Height)

|  | Max Height (ft) | Key | Max Height, First Story (ft) | Key |
| --- | --- | --- | --- | --- |
| Principal Structure | 60 | A | n/a | B |
| Accessory Structure(s) | 25 | C | n/a | D |

- Table 5.05.06 Access.

Access standards are still under development in coordination with updates to the Knox County Access Policy. This section will be updated following completion of draft Articles 13 and 18 and further guidance provided by policy adjustment.

- Table 5.05.07 Parking. (Figure 5.05.04 Parking)

| Surface Parking | Surface Parking | Surface Parking |
| --- | --- | --- |
| Location | Permitted (y/n) | Key |
| Front yard | Y* | A |
| Side yard (interior) | Y | B |
| Side yard (street) | Y | C |
| Rear yard | Y | D |
| Minimum Parking Setback from Property Boundary (ft) | Minimum Parking Setback from Property Boundary (ft) | Minimum Parking Setback from Property Boundary (ft) |
| Front^ | 25* | A |
| Side (interior) | 10 | B |
| Side (street) | 12 | C |
| Rear^ | 10 | D |

^Does not apply to driveway surface parking.

*Landscaping shall be required in accordance with Article 14 in the minimum parking setback.

- Infrastructure.

No additional infrastructure requirements have currently been identified. Moving forward, this section will be used to point to specific requirements if they are needed.

- Landscaping.

No district-specific infrastructure requirements have currently been identified. Moving forward, this section will be used to identify district-specific requirements if they are needed.

- Open Space.

- The required open space set-aside in an BC zoning district can be found in Article 16 of this ordinance.

- Additional Considerations.

No additional considerations have currently been identified. Moving forward, this section will be used to point to specific requirements if they are needed.

## 5.06	EC Employment Campus Zoning District

- Purpose: The employment campus zoning district provides for the development of business, technology, and office parks that accommodate a mix of uses that are traditionally viewed as employment-generating, such as research and development, light manufacturing, medical and professional office complexes, and related uses and activities. Site design and architectural standards ensure these districts are compatible with adjacent uses and districts, while also creating an attractive campus-style environment for major employers. Development in this district should include a high level of amenities for employees and visitors, including open space, pedestrian connections, and shared facilities.

- Typical Development Pattern.

- Uses and Activities Allowed.

      - Uses and activities allowed in the EC zoning district are established in Article 9 of this ordinance as permitted or conditionally permitted. Reference the consolidated table of uses for additional information.

For ease in review of these articles, a temporary list of permitted and conditional uses allowed within the EC zoning district has been provided in Table 5.06.01 below.

      - Multiple principal and accessory uses and structures may be permitted on a lot or development site within this zoning district. Multiple uses may be allowed within a single structure.

- 5.06.02 Lot Dimensions. (Figure 5.06.01 Lot Dimensions)

|  | Min. Lot or Development Site (ac) | Gross Floor Area (sq. ft.) | Min. Lot or Development Site Width (ft) | Width to Depth Ratio (%) | Max. Impervious Surface (%) |
| --- | --- | --- | --- | --- | --- |
| All types | 3 | - | 100 | n/a | 70 |
| Key | C | D | A | A/B | - |

- Table 5.06.03 Lot Setbacks. (Figure 5.06.02 Lot Setbacks and Build-To Zone)

|  | Min. Front (ft) | Min. Rear (ft) | Min. Side, Interior (ft) | Min. Side, Street (ft) | Interior (ft. btwn buildings if multiple) |
| --- | --- | --- | --- | --- | --- |
| Principal Structure(s) | 10* | 15^ | 12^ | 10* | 24 |
| Accessory Structure(s) | 10* | 5^ | 5^ | 10* | 24 |
| Key | A | B | C | D | E |

^Setback increases to 20 feet if abutting a residential zoning district or lot with an established residential use.

*Street frontage type may dictate a reduced setback; refer to Article 18 for additional detail.

- Table 5.06.04 Build-to Zone. (Figure 5.06.02 Lot Setbacks and Build-To Zone)

Established build-to zones apply to internal roads within a development site.

|  | BTZ - Front (ft) | Front (%) | Key | BTZ – Side Street (ft) | Side Street (%) | Key |
| --- | --- | --- | --- | --- | --- | --- |
| Principal Structure(s) | 15 | 40 | F | 20 | 30 | G |
| Accessory Structure(s) | None established | n/a | F | None established | n/a | G |

- Table 5.06.05 Building Height. (Figure 5.06.03 Building Height)

|  | Max Height (ft) | Key | Max Height, First Story (ft)^ | Key |
| --- | --- | --- | --- | --- |
| Principal Structure(s) | 60 | A | 15 | B |
| Accessory Structure(s) | 25 | C | n/a | D |

^Applies to office and retail/commercial uses and structures only.

- Table 5.06.06 Access.

Access standards are still under development in coordination with updates to the Knox County Access Policy. This section will be updated following completion of draft Articles 13 and 18 and further guidance provided by policy adjustment.

- Table 5.06.07 Parking.

| Surface Parking | Surface Parking | Surface Parking |
| --- | --- | --- |
| Location | Permitted (y/n) | Key |
| Front yard | Y | A |
| Side yard (interior) | Y | B |
| Side yard (street) | Y | C |
| Rear yard | Y | D |
| Minimum Parking Setback from Property Boundary (ft) | Minimum Parking Setback from Property Boundary (ft) | Minimum Parking Setback from Property Boundary (ft) |
| Front | 15 | A |
| Side (interior) | 15 | B |
| Side (street) | 15 | C |
| Rear | 15 | D |

- Infrastructure.

No additional infrastructure requirements have currently been identified. Moving forward, this section will be used to point to specific requirements if they are needed.

- Landscaping.

      - A unified landscaping plan for the development is required, pursuant to the standards established in Article 14 of this ordinance.

- Open Space.

      - The required open space set-aside in an EC zoning district can be found in Article 16 of this ordinance.

- Additional Considerations.

  - Overall campus site plan required (TBD)

  - Unified sign plan required (TBD)

    - Develop consistent sign requirements for EC, keep to one monument by development, each business allocated by building - look at examples for establishing sign plan for large-scale development sites

    - Allow for two signs (if excessive sq. ft.) - allow for so much sign area for each business, once you reach a max can create multiple (limit height)

## 5.07	TC - Town Center Zoning District

- Purpose: The town center zoning district is intended to create compact, walkable, mixed-use hubs through a blend of services, shops, restaurants, a mix of housing types, and public spaces. Uses may be both vertically and horizontally integrated and be located on one parcel or across multiple parcels and develop at different scales to align with the surrounding development pattern. Development is subject to site design standards and performance criteria that support streetscape design and the integration of public spaces and amenities throughout.

- Typical Development Pattern.

- Uses and Activities Allowed.

      - Uses and activities allowed in the TC zoning district are established in Article 9 of this ordinance as permitted or conditionally permitted. Reference the consolidated table of uses for additional information.

For ease in review of these articles, a temporary list of permitted and conditional uses allowed within the TC zoning district has been provided in Table 5.07.01 below.

      - Multiple principal and accessory uses and structures may be permitted on a lot within this zoning district. Multiple uses may be allowed within a single structure.

- Table 5.07.02 Lot Dimensions. (Figure 5.07.01 Lot Dimensions)

| Building Type | Min. Lot Area (sf) | Min. Lot Area (sf) w/Alley | Min. Lot Width (ft) | Min. Lot Width (ft) w/Alley | Width to Depth Ratio | Max. Impervious Surface (%) |
| --- | --- | --- | --- | --- | --- | --- |
| Mixed-use vertical | No minimum | No minimum | 30 | 30 | n/a | 90 |
| Key | C | C | A | A | A/B | - |

- Table 5.07.03 Lot Setbacks. (Figure 5.07.02 Lot Setbacks and Build-To Zone)

| All building types | Min. Front (ft) | Min. Rear (ft) | Min. Side, Interior (ft) | Min. Side, Street (ft) | Interior (ft. btwn buildings if multiple) |
| --- | --- | --- | --- | --- | --- |
| Principal Structure(s) | 0 | 0 | 0 | 5 | 0 |
| Accessory Structure(s) | 0 | 0 | 0 | 5 | 0 |
| Key | A | B | C | D | E |

- Table 5.07.04 Build-to Zone. (Figure 5.07.02 Lot Setbacks and Build-To Zone)

| All Building Types | BTZ - Front (ft) | Front (%) | Key | BTZ – Side Street (ft) | Side Street (%) | Key |
| --- | --- | --- | --- | --- | --- | --- |
| Principal Structure(s) | 5 | 70 | F | 5 | 50 | G |
| Accessory Structure(s) | None established | n/a | F | 5 | n/a | G |

- Table 5.07.05 Building Height.  (Figure 5.07.03 Building Height)

|  | Max Height (ft) | Key | Max Height, First Story | Key |
| --- | --- | --- | --- | --- |
| Principal Structure(s) | No maximum | A | 15 | B |
| Accessory Structure(s) | 25 | C | 15 | D |

- Table 5.07.06 Access.

Access standards are still under development in coordination with updates to the Knox County Access Policy. This section will be updated following completion of draft Articles 13 and 18 and further guidance provided by policy adjustment.

- Table 5.07.07 Parking. (Figure 5.07.04 Parking)

| Surface Parking | Surface Parking | Surface Parking |
| --- | --- | --- |
| Location | Permitted (y/n) | Key |
| Front yard | N | A |
| Side yard (interior) | N | B |
| Side yard (street) | N | C |
| Rear yard | Y | D |
| Minimum Parking Setback from Property Boundary (ft) | Minimum Parking Setback from Property Boundary (ft) | Minimum Parking Setback from Property Boundary (ft) |
| Front | n/a | A |
| Side (interior) | n/a | B |
| Side (street) | n/a | C |
| Rear | 5 | D |

- Infrastructure.

No district-specific infrastructure requirements have currently been identified. Moving forward, this section will be used to identify district-specific requirements if they are needed.

- Landscaping.

No district-specific landscaping requirements have currently been identified. Moving forward, this section will be used to identify district-specific requirements if they are needed.

- Open Space.

      - There is norequired open space set-aside in the TC zoning district.

- Additional Considerations.

      - Front-facing garages are not permitted in the TC zoning district.